//week02-1-void-setup-void-draw-fill-textsize-text-key

//鍵盤的操作 與上週的mouse結合

//File-Preference 字型放大

void setup(){

size(500, 500); //視窗大小

}

void draw(){

if(mousePressed) background(#F58A8A);

else background(#D6F074);//tool-color選擇器

fill(0, 0, 255);//藍色的填充色

textSize(80); //字的大小

text("key: "+ key, 200, 300);

}

打開桌面資料夾 2024葉正聖老師上課資料

開啟CH341SER.EXE

Install 安裝

桌面windows右鍵選裝置管理員

選連接埠 看USB-SERIAL CH340(看這裡是COM多少)

選工具連接埠 選跟剛剛一樣的數字COM

//week02-2-arduino-void-setup-void-loop-digitalwrite

//人生中第一個arduino程式

void setup() {

// put your setup code here, to run once:

pinMode(8, OUTPUT); //第8個腳 要發出buzzer聲音

}

//勾勾(CTRL-R編譯程式)箭頭往右(CTRL-R上傳到電路板)

void loop() {

// put your main code here, to run repeatedly:

digitalWrite(8, HIGH); //發出高電位

delay(1000); //等1秒(1000ms = 1sec 1ms = 0.001sec)

digitalWrite(8, LOW); //發出低電位

delay(1000); //等1秒(1000很慢 10很吵 2有點聲音 1音頻很高)

}

選makeruno defaultsketch

//week02-3-arduino-makeruno-github-dafault-sketch

//google: makeruno github 找到 CytronTechnology 官網的 default sketch

#define NOTE_C5 523

#define NOTE_E5 659

#define NOTE_G5 784

#define BUTTON 2

#define BUZZER 8

int melody[] = {

NOTE_E5, NOTE_E5, 0, NOTE_E5, 0, NOTE_C5, NOTE_E5, 0, NOTE_G5

};

int noteDurations[] = {

10, 10, 10, 10, 10, 10, 10, 10, 10

};

int pin;

int ledArrayHigh;

int ledArrayLow;

boolean mode = false;

boolean buttonPressed = false;

void setup()

{

delay(1000);

pinMode(BUTTON, INPUT_PULLUP);

for (pin = 3; pin < 14; pin++) {

pinMode(pin, OUTPUT);

}

for (int thisNote = 0; thisNote < 9; thisNote++) {

int noteDuration = 1000 / noteDurations[thisNote];

tone(BUZZER, melody[thisNote], noteDuration);

int pauseBetweenNotes = noteDuration * 1.30;

delay(pauseBetweenNotes);

noTone(BUZZER);

}

}

void loop()

{

for (pin = 0; pin < 5; pin++) {

if (digitalRead(BUTTON) == LOW &&

buttonPressed == false) {

buttonPressed = true;

mode = !mode;

pin = 0;

if (mode == false) {

tone(BUZZER, NOTE_C5, 100);

delay(100);

tone(BUZZER, NOTE_G5, 100);

delay(100);

noTone(BUZZER);

}

else if (mode == true) {

tone(BUZZER, NOTE_G5, 100);

delay(100);

tone(BUZZER, NOTE_C5, 100);

delay(100);

noTone(BUZZER);

}

}

if (mode == false) {

ledArrayHigh = 13 - pin;

ledArrayLow = 7 - pin;

}

else if (mode == true) {

ledArrayHigh = 9 + pin;

ledArrayLow = 3 + pin;

}

digitalWrite(ledArrayHigh, HIGH);

digitalWrite(ledArrayLow, HIGH);

delay(100);

digitalWrite(ledArrayHigh, LOW);

digitalWrite(ledArrayLow, LOW);

if (pin == 4) delay(100);

}

if (buttonPressed == true) {

buttonPressed = false;

}

}

//week02-4-arduino-tone

void setup() {

// put your setup code here, to run once:

pinMode(8, OUTPUT);

//上面setup只做一次

tone(8, 523, 1000); // Do 0.1sec

delay(1000);

tone(8, 587, 1000); // Re 0.1sec

delay(1000);

tone(8, 659, 1000); // Me 0.1sec

delay(1000);

}

void loop() {

// put your main code here, to run repeatedly:

//下面會一直迴圈重複做 不會停

}

//week02-5-arduino-do-re-me-serial-begin-available-read-if-tone

//把arduino跟processing結合

//在processing按下1 2 3 對應do re mi使用usb serial

//寫完程式 用工具tool-序列埠監控視窗serial monitor 來傳送1 2 3 測試很麻煩

void setup() {

Serial.begin(9600); //usb serial 開始轉動 轉速 9600 bps

}

void loop() {

if (Serial.available()){ // 如果usb serial有收到資料

char c = Serial.read(); //讀近來

if (c=='1') tone(8, 523, 1000); // Do 0.1sec

if (c=='2') tone(8, 587, 1000); // Re 0.1sec

if (c=='3') tone(8, 659, 1000); // Me 0.1sec

}

}

//week02-5-processing-do-re-mi-import-serial-myport--void-keyPressed-write

//把arduino跟processing結合

//在processing按下1 2 3 對應do re mi使用usb serial

import processing.serial.*;

Serial myPort;

void setup(){

myPort = new Serial(this, "COM3", 9600);

}

void draw(){

}

void keyPressed(){

if(key==&apos;1&apos;) myPort.write(&apos;1&apos;);

if(key==&apos;2&apos;) myPort.write(&apos;2&apos;);

if(key==&apos;3&apos;) myPort.write(&apos;3&apos;);

}